LAB 3: 
Using the AXI Hardware Timer

COEN 317 - Microprocessor-Based Systems – Fall 2025

Section FL - X

Lab performed on : October 30th 2025

Report submitted on : November 13th 2025

Lab performed and report written by : Simon Guindon – 40058301

Objectives

During this lab, we will learn how to use the LogiCORE IP AXI Timer. This timer possesses two counters and can be used in various modes. The goal is to observe how the timer can be used in its different modes and how we can interface the timers with the ZC702 Board.

Theory

The main theory comes from understanding the different modes of operation of the timer. For the first part of the lab, we will use the AXI timer as a basic timer and counter. The code that we will run will start the timer before a command and stop the timer after a command so we can see how many clock cycles are required to run a command in C++ for our hardware. In this case, we wanted to see how to perform XGpio_DiscreteWrite(). We started by creating an embedded processor in planAhead that will contain an instance of AXI GPIO and a AXI Timer/Counter. For the second part of the lab, we will use the timer in Capture mode. The main goal here was to familiarize the experimenter with the switch bounce that happens when the hardware captures a physical input from the user (button on the board). Finally, the third part was to create a PWM signal where both timers will be used to create a signal of a specific length (Tracked by Timer1) with a specific period (Tracked by Timer0)

Questions

- Upon changing the values in the Boolean arrays A and B and recompiling the code and downloading it to the board, we obtained several LEDs turning on, depending on if both arrays had ‘1’ for a given position. For instance, having both arrays be filled with 1s, we get all LEDs to turn on (See Appendix Figure 1 and 2). Also, having 1 0 1 0 1 0 1 0 for both resulted in LED 0 2 4 and 6 turning on (See Appendix Figure 3 and 4). Finally, having 1 1 1 1 0 0 0 0 or both arrays resulted in LEDs 0 1 2 3 to be on (see figure 5 6 in Appendix).  This is consistent with what we expected from an AND gate behavior.

Conclusions

In this lab, our objective was to program our Xilinx ZC702 development board with a AXI Timer and Counter. We managed to use the Timer in three different situations (Timing software, getting physical feedback from the user and creating a PWM signal of desired length). The next lab will build upon the theory learned here to created more complicated systems.

Appendix